The last lecture, and we shall also discuss digital signals, in the last lecture we talked:

~Course contents

~Books recommended

~Definition of a signal.

: A signal is a function of one or more variables and accordingly it can be 1D 2D, Multi-Dimensional.

~Signals: Analog (CT, DT (becomes Digital signals when pass through A/D converter)): In DT, is Discrete time signal is not a digital signal; a discrete time signal is one in which the amplitude is a continuum, and not discretized. On the other hand, if the time is discretized, this signal is still analog. Only when a discrete time signal is passed through an A to D converter, it becomes a digital signal.

~in analog domain: we have two kinds of signals; continuous time and discrete time and we

~also emphasized the need for processing: Why should one process a signal? This is to obtain a better quality signal than the original one. It could be for example,

~Examples:

Noise filtering or improving the quality of a picture in terms of brightness or contrast or whatever it is.

===================================================================

~Signal processing can be three types:

Analog	2. Digital 	3. Mixed

~Current trend is to make mixed signal processing.

~Why digital signal processing? Why can’t we do analog signal processing only? Because in practice signals are analog. But DSP should be the natural choice if the signal is digital, for example!

: Rainfall data, Time is discretized: 1980, 1981, and 1982 etc and there is nothing between 80 and 81, there nothing like 1980.5 or 1980.41.

So, the natural choice would be DSP and this is what is done.

What is DSP?

Addition, Multiplication and Delay or recalling of past signals are the basic operations in DSP. Even if the signal is analog, one prefers to use DSP.

The reasons are many. DSP has many advantages over ASP or Analog signal processing and many a times it happens to Lecture 2 - Digital Signal Processing Introduction Contd - YouTube.

Be a less costly proposition. Digital IC’s are available off the shelf at very low cost.

There are basically three operations: Delay, Multiplication, and Addition. So this is what the Digital Processor does. It is very simple device, it simply performs 3 operations. Multiplication is repeated addition, only two signals at a time; subtraction is also a form of addition with one of these signals negated or the sign bit changed.

So, there is no integration, there is no differentiation; there are no complicated operations which limits the usefulness of analog processing. The output of the digital processor is also digital, A sequence of numbers, and in order that output will be useful, you must convert it back to analog form. So you have a D to A or Digital to Analog converter. The output of the digital to analog converter is in the form of stair cases.

===================================================================

The advantages that accrue out of the use of DSP Digital signal processing, as compared to ASP Analog signal processing are many:

First, the digital signal processor is less sensitive to component tolerances and environmental changes. In analog signal processing change in any element, an inductance or a capacitance, or a small shift in the power supply causes a change in the performance. This also happens in digital circuits but since you are coding high amplitude by 1 and low amplitude by 0, even if the amplitude changes by 10% it does not matter, because 1 remains 1 and 0 remains 0. Unless the amplitude drops down to the noise margin level you have to nothing to worry. Therefore digital circuits are less sensitive to component tolerances and environmental changes.

Digital circuits allow” volume production without the need for adjustment during construction or use”. Once the process is set, the timings and the steps, nothing has to be changed.

Digital circuits are of course amenable to full integration. This is not possible for analog circuits because of difficulty of integrating inductances and transformers. A transformer cannot be made into an integrated chip; same is the case with an inductance of a respectable value. Inductances of very small values (Pico-Henry) can be made by IC’s. But an inductance of a respectable value like micro- henry or milli – henry is very difficult to make because inductance basically requires a large volume. Inductance is flux per unit current. In order to generate a flux you require space. You cannot generate a large amount of flux in a small space unless the magnetic permeability (Magnetic permeability is the ability of a material to respond to how much electromagnetic flux it can support to pass through itself within an applied electromagnetic field. In other word magnetic permeability of a material is the degree of magnetization capability. Magnetic permeability is expressed in μ that is a Greek Letter.) is astronomically high. Very high permeability materials are not available and therefore analog circuits are not amenable to full integration; Inductances and Transformers have to be outside the chip.

In digital signal processing, accuracy can be increased almost indefinitely by increasing the word length (the number of bits in a word). Of course, cost also rises; as you put more number of bits you have to pay more but the accuracy increasing. In an analog signal processor the dynamic range (Dynamic   range (abbreviated DR, DNR, or DYR) is the ratio between the largest and smallest values that a certain quantity can assume. It is often used in the context of signals, like sound and light. It is measured either as a ratio or as a base-10 (decibel) or base-2 (doublings, bits or stops) logarithmic value of the difference between the smallest and largest signal values.))

Is limited by the power supply. In an operational amplifier, OP AMP, for example, the output cannot exceed +Vcc or –Vcc; it gets saturated. In digital signal processing since we are handling numbers there is no question of saturated (put a device into a state in which no further increase in current is achievable). We can handle as large numbers as possible and if fixed point does not do the job, use floating point with characteristic and mantissa stored in separate registers. You can then achieve very large dynamic range. This is not possible in analog signal processing. The same digital signal processor can do multiple jobs. For example, use the same processor to time share two different operations and the following picture shows, how this is done.

Time multiplexing is one of the great advantages of DSP; you cannot do it with Analog signal processing. If you mixed two signals continuous time or discrete time signals, there is no way you can separate them.

~Continuing the advantages of DSP.

A digital signal processor can be very easily adjusted for different characteristics by simply changing some numbers, which form the coefficients. This can also be done in analog signal processing by varying a resistor, or a capacitor or an inductor but changing a number is much simpler then changing R, L or C. Varying an inductance for example is very elaborate procedure. Varying a capacitance is not a problem but then you have problems of contact. You also have problems of environmental changes and so on and so forth. Similarly for resistors, you know that the potentiometer contact gradually wears off due to mechanical friction. There is nothing like this in DSP, you simply change a number which is one of the coefficients and then you get a different characteristics. This is required in adaptive filters or variable cutoff filters.

One of the major and dominant advantages of DSP is that exact linear phase systems can be designed. This is not possible in analog signal processing and has been a dream for analog communication engineers to get Exact Linear Phase means there is no delay distortion. We shall come back to the importance of linear phase in a later lecture. But this is one of the major advantages of DSP that it can attain exact linear phase and therefore it permits processing without any delay distortion.

One of the advantages of DSP is also that different parts of the signal processor can work at different rates. That is, the sampling rate can be different in different parts of the processor. There are well established techniques for changing one sampling one sampling rate to another. You can sample up or sample down without any problem. This facility is not possible in analog signal processing.

When two amplifiers are cascaded (arrange (a number of devices or objects) in a series or sequence), ideally the overall gain is the product of the two gains. However, this is not invariably the case in practice because one amplifier loads the other. As soon as you, put a load speaker at the output of your power amplifier, unless it is perfectly matched, the power amplifier output goes down. There is no such problem in cascading two or more digital circuits provided you do not exceed the fan out capability of the circuits. In short, there are no loading problems due to cascading in digital circuits. Digital signals can easily be stored in magnetic tapes and disks, including optical disks, as long as one likes. There will be no problem in storage.

In DSP, very low frequency signal processing is possible with ease. But in analog signal very low frequency signal processing is possible with ease. But in analog signal processing, Inductance poses a problem. There is no such problem is digital signal processing at very low frequencies. However, all are not roses with digital signal processing, there are disadvantages also. The major disadvantage, of course, is increased complexity. You have to use more devices: sample and hold, A to D, D to A, and Low pass filters etc. another,

Major disadvantage is that the frequency range for operation of a DSP is limited due to the limited rate at which you can sample and hold. Also in A to D conversion, the speed is limited. In practice you cannot use a sampling frequency, with today’s state of the art, which is more than 10 megahertz and therefore if the sampling frequency is 10 megahertz, naturally signal processing cannot be done for more than 5 megahertz.

The other disadvantages is that, if you have a purely passive circuit consisting of Inductance, resistance and capacitance to do the analog signal processing, there is power dissipation only in the resistance, but it is much less compared to the power drawn by digital circuits, main dissipation being in the active devices, the transistors. A typical AT and T DSP32C processor which contains 405,000 transistors dissipates 1 watt. There are no such problems with purely LC circuits. But even then one does use DSP because of its dominant advantages.

Power dissipation being one of the major challenges for DSP engineers.

One of the aims of TI Texas Instruments is to make digital circuits which will require no power supply, particularly for “devices which are to be implanted in live bodies like the human body.”

--------------------------------------------------------------------------------------------

Digital signals:

A digital signal is obtained from an analog signal by sampling. What you get here after sampling xat→xant.

Intervals of T is xant;this is not a digital signal but an analog discrete time signal. It exists only at capital T, 2T and so on. But its amplitude can be a continuum, then you feed it to an A to D and what you get at the output is a truly digital signal for which we give the symbol xn; the subscript A is lost because it becomes digital and capital T in the process is also lost, in the sense that when small n becomes a variable, it can take integer values only – positive or negative and each integer value of n in the corresponding discrete time signal corresponds to the integer multiplied by the sampling interval. So T is lost in the digital signal but if this signal was derived from an analog signal, then T has to be kept in mind. One way of interpreting x(n) is that capital T has been normalized to 1. However, it is a beauty of DSP and Discrete time signal processing (DTSP) that the mathematical techniques for handling discrete time signals and digital signals are the same. They are: difference equations, Z – transforms, Fourier transforms and discrete Fourier transforms. So whether the dependent variable is a continuum, like xanT, or of discrete amplitude, likex(n), the techniques are the same and therefore we shall use xanT and x(n) interchangeably. We shall always take that the signal has been discretized in time as well as amplitude. This sampling interval T is obviously the reciprocal of sampling frequency fs. we shall see later that our signal processing must be limited to half of the sampling frequency, we cannot go beyond that. Obviously then, digital signal is a sequence of numbers and the sequence sometimes is simply represented as x(n) where n takes values theoretically from – infinity to + infinity but at intervals of 1. To show that it is a sequence we sometimes give the symbol of x(n) within curly brackets xn.

How is n indicated? N=0 is indicated, through an arrow and one writes n=0 below it; in the firstx(n), signal x(n) is equal!

~Sequence:

Can be even, odd, mix i.e. general.

n=-KKxn2=Average Power..

Pav≜1N. n=0Say N-1xn2

n=N it starts repeating because it is a periodic signal and then divide by N.

======================================================